📋 BeeLine Moto II Clone — MVP Product Specifications Document

Document Version: 1.0.0

Project Codename: BeeLine-Clone (Moto II Architecture)

Target Platform: ESP32-S3 Round Display (412×412 IPS) + Android Companion App (Ola Maps SDK)

1. Executive Summary & Product Vision

1.1 Product Overview

The BeeLine Clone is a minimalist, glanceable motorcycle and bicycle smart navigation display inspired by the BeeLine Moto II. It solves the primary pain points of motorcycle navigation: distraction, phone overheating, vibration camera damage, and battery drain from keeping smartphone screens active in direct sunlight.

The system decouples heavy compute (routing, GPS, map tiles, traffic) to the smartphone companion app in the rider's pocket/mount, and streams lightweight, glanceable vector geometry and HUD telemetry over Bluetooth Low Energy (BLE) to an ultra-bright, compact 1.46" round display mounted on the handlebars.

1.2 Core MVP Philosophy

Glanceable Safety: Information must be readable in < 0.5 seconds at speed.

Vector Abstraction, Not Video/Tiles: No heavy raster images or map tile decoding on the microcontroller. The companion app projects road geometry into local rider-centric vector coordinates; the display renders crisp hardware-accelerated vectors at 50 FPS.

Ultra-Low Latency & Bandwidth: Navigation updates stream at 5 Hz (200 ms interval) in single BLE packets (< 128 bytes), ensuring zero frame drops and minimal battery impact.

2. System Architecture

┌──────────────────────────────────────────────────────────────────┐ │ SMARTPHONE COMPANION APP │ │ (React Native / Expo / Android 14) │ │ │ │ ┌────────────────────┐ ┌────────────────────────┐ │ │ │ Ola Maps API │ │ Real-Time GPS Service │ │ │ │ (Directions/Geocode│ │ (Fused Location 10Hz) │ │ │ └─────────┬──────────┘ └───────────┬────────────┘ │ │ │ │ │ │ ▼ ▼ │ │ ┌──────────────────────────────────────────────────────────┐ │ │ │ Heading-Up Vector Projection Engine │ │ │ │ - 280m Lookahead Window │ │ │ │ - World Lat/Lon ➔ Polar/Screen (X: 0..412, Y: 0..220) │ │ │ │ - Exit & Cross-Street Vector Branch Extraction │ │ │ └─────────────────────────┬────────────────────────────────┘ │ │ │ Base64 Telemetry Stream (5 Hz) │ └─────────────────────────────┼────────────────────────────────────┘ │ BLE 5.0 Wireless Link (GATT Service: 1a2b3c4d-5e6f-7a8b-9c0d-1e2f3a4b5c6d) │ ┌─────────────────────────────▼────────────────────────────────────┐ │ EMBEDDED DISPLAY CONTROLLER │ │ (ESP32-S3R8 + Waveshare 1.46" Round IPS) │ │ │ │ ┌──────────────────────────────────────────────────────────┐ │ │ │ NimBLE GATT Server (MTU: 185 Bytes) │ │ │ │ - NAV_STATE_CHAR (Write Without Response) │ │ │ │ - DEVICE_EVENT_CHAR (Notify on Button Push) │ │ │ └─────────────────────────┬────────────────────────────────┘ │ │ │ Binary Unpack (Zero Allocation) │ │ ▼ │ │ ┌──────────────────────────────────────────────────────────┐ │ │ │ LVGL 8.4 Rendering Pipeline (50 FPS, PSRAM Buffers) │ │ │ │ - Region 1: Vector Road Network & Junction Manoeuvre │ │ │ │ - Region 2: Distance Value, Unit, Speed Limit Sign │ │ │ │ - Region 3: Trip Progress Rim Arc │ │ │ └─────────────────────────┬────────────────────────────────┘ │ │ │ QSPI Quad-Bus (40 MHz) │ │ ▼ │ │ ┌──────────────────────────────────────────────────────────┐ │ │ │ SPD2010 412×412 Round IPS Full-Color Panel │ │ │ └──────────────────────────────────────────────────────────┘ │ └──────────────────────────────────────────────────────────────────┘

3. Hardware Specifications

3.1 Microcontroller & Core Board

MCU: Espressif ESP32-S3R8 (Dual-core Xtensa LX7 @ 240 MHz, vector instructions enabled)

Flash Memory: 16 MB QIO SPI Flash

Internal RAM: 512 KB SRAM + 8 MB Octal SPI PSRAM (OPI)

Wireless: 2.4 GHz Wi-Fi 4 + Bluetooth 5.0 (LE)

Platform Module: Waveshare ESP32-S3-Touch-LCD-1.46B

3.2 Display Subsystem

Panel: 1.46-inch Round IPS LCD

Resolution: 412 × 412 pixels (~354 PPI, circular clip)

Color Depth: RGB565 (16-bit, big-endian byte order)

Display Driver IC: SPD2010

Host Interface: High-Speed QSPI (Quad-SPI) running at 40 MHz

Backlight Control: Hardware LEDC PWM @ 5 kHz on GPIO 5 (0–100% brightness control)

3.3 Pinout Mapping & Peripheral Bus

| Signal Name | ESP32-S3 GPIO | Interface / Bus | Description |
| --- | --- | --- | --- |
| LCD_SCK | GPIO 40 | QSPI | SPI Clock (40 MHz) |
| LCD_CS | GPIO 21 | QSPI | Display Chip Select (active LOW) |
| LCD_SDA0 | GPIO 46 | QSPI | Data Line 0 |
| LCD_SDA1 | GPIO 45 | QSPI | Data Line 1 |
| LCD_SDA2 | GPIO 42 | QSPI | Data Line 2 |
| LCD_SDA3 | GPIO 41 | QSPI | Data Line 3 |
| LCD_TE | GPIO 18 | Input | Tearing Effect Sync Input |
| LCD_BL | GPIO 5 | LEDC PWM | Backlight Control |
| I2C_SDA | GPIO 11 | I2C (400 kHz) | Shared I2C Bus Data Line |
| I2C_SCL | GPIO 10 | I2C (400 kHz) | Shared I2C Bus Clock Line |
| BOOT_KEY | GPIO 0 | GPIO (Pull-Up) | Physical Input Button (Short/Long Press) |

3.4 I2C Expander & Power Subsystem

I2C Expander: TCA9554PWR (I2C address 0x20)

EXIO1: Touch Reset (TP_RST)

EXIO2: LCD Hardware Reset (LCD_RST)

EXIO3: MicroSD Chip Select (SD_CS)

Touch Controller: SPD2010 Capacitive Touch (Intentionally detached in MVP firmware to maximize battery life and dedicated I2C bandwidth for sensors).

Auxiliary Sensors (Available on I2C bus):

IMU: QMI8658 6-Axis Accelerometer/Gyroscope

RTC: PCF85063 Real-Time Clock

4. Display Layout & UI Design Specifications

The 412×412 circular display is divided into three distinct visual regions optimized for high-contrast, glanceable readability under direct sunlight:

(0,0) ┌─────────────────────────────────────────────────────────────┐ (412,0) │ [ STREET BANNER: "GRAND AVENUE" (180x26px Pill) ] │ │ │ │ REGION 1: VECTOR MINI-MAP │ │ (Y: 0px to 220px) │ │ │ │ - Background: Deep Obsidian `#0E0F14` │ │ - Active Route: Dual-Layer Polyline │ │ * 14px Dark Casing `#1B1C24` │ │ * 8px Crisp Paved White Core `#FFFFFF` │ │ - Junction Branches: 4px Semi-Transparent Light Gray │ │ vectors (`#CCCCCC` @ 55% opacity) │ │ - Rider Pointer: White Isosceles Triangle (▲) at │ │ fixed origin `(206, 210)` pointing upward │ │ │ ├─────────────────────────────────────────────────────────────┤ │ │ │ REGION 2: MANOEUVRE & HUD PANEL │ │ (Y: 220px to 345px) │ │ │ │ [Left: X 104..161] [Center: X 195] │ │ - Turn Icon: ⤷ (9px Vector) - Distance: "300" │ │ Smooth 90° Fillet Curve Montserrat Bold 48 │ │ - Unit: "m" │ │ [Right: X 295, Y 220] Montserrat SemiBold 32 │ │ - Speed Badge: 60×60px Circle │ │ Border: `#FF3B30` (5px), Fill: `#FFFFFF`, Text: "70" │ │ │ ├─────────────────────────────────────────────────────────────┤ │ REGION 3: PROGRESS RIM ARC │ │ (Y: 345px to 412px) │ │ - Arc Diameter: 390px, Center: (206, 206) │ │ - Angle Range: 35° to 145° (110° Arc on Lower Rim) │ │ - Line Thickness: 10px Pure White `#FFFFFF` │ │ - Dynamics: Fills proportionally with trip progress │ (0,412)└─────────────────────────────────────────────────────────────┘ (412,412)

4.1 Color Palette & Design Tokens

| UI Element | Color Name | Hex Code | Visual Purpose |
| --- | --- | --- | --- |
| Canvas Background | Deep Obsidian | #0E0F14 | High contrast, zero glare |
| Active Route Casing | Dark Asphalt | #1B1C24 | Separates active path from background |
| Active Route Core | Signal White | #FFFFFF | Core path to follow |
| Junction Branches | Arterial Gray | #CCCCCC | Upcoming turns and crossing streets |
| Speed Limit Ring | Regulatory Red | #FF3B30 | Official European/Indian speed badge |
| Speed Limit Text | High-Contrast Black | #000000 | Legibility inside speed badge |
| Distance & Unit | Bright White | #FFFFFF | Immediate readability at distance |
| Street Name Banner | Translucent Slate | #141620 (80%) | Non-intrusive road identification |

5. Companion App Specifications

5.1 Architecture & Stack

Framework: React Native 0.74+ with Expo (Bare workflow with prebuild)

Language: TypeScript (Strict Mode)

BLE Client: react-native-ble-plx

Maps & Routing Engine:

Map UI: MapLibre Native SDK for Android

Directions & Geocoding: Ola Maps REST API (India-optimized routing)

Location Service: Fused Location Provider (Android Native LocationManager)

5.2 Dynamic Vector Coordinate Projection Engine

Instead of streaming full road shapes or high-bandwidth tile images, the companion app continuously performs 2D mathematical coordinate projection from real-world GPS coordinates to the device's screen space:

Rider Origin: Fixed at screen bottom-center (206, 210).

Heading Orientation: Rotates all upcoming GPS coordinates by -currentHeading so the rider's forward travel vector points strictly upward (-Y axis).

Lookahead Horizon: Dynamically clips road coordinates to a 280-meter forward window.

Exit & Cross-Street Extraction:

Extracts the current road segment and the active maneuver exit arm.

Extracts up to 3 intersecting crossing streets/branches and converts them to screen vector lines (x1, y1) ➔ (x2, y2).

Payload Compression: Encodes waypoints and branches into a compact binary byte array and transmits over BLE via Write Without Response (WRITE_NR).

6. BLE Telemetry Protocol Specification

6.1 GATT Service & Characteristic UUIDs

Service UUID: 1a2b3c4d-5e6f-7a8b-9c0d-1e2f3a4b5c6d

Nav State Characteristic (Phone ➔ Device): 1a2b3c4d-5e6f-7a8b-9c0d-1e2f3a4b5c6e (Property: WRITE_NR)

Device Event Characteristic (Device ➔ Phone): 1a2b3c4d-5e6f-7a8b-9c0d-1e2f3a4b5c6f (Property: NOTIFY)

6.2 Nav State Telemetry Packet Structure (Binary)

The companion app sends a variable-length binary packet (12 to ~80 bytes) at 5 Hz (200 ms):

[0..11] Mandatory Header (12 Bytes) ├── Byte 0: turn_type (uint8) ├── Byte 1-2: distance_m (uint16 LE) ├── Byte 3: speed_limit_kph (uint8) ├── Byte 4-5: eta_min (uint16 LE) ├── Byte 6: flags (uint8: bit 0 = metric, bit 1 = has_vector_path) ├── Byte 7: trip_progress_pct (uint8, 0..100) ├── Byte 8: side_road_y_offset (uint8) ├── Byte 9: poi_type (uint8) ├── Byte 10: poi_x_rel_m (int8) └── Byte 11: poi_y_rel_m (int8) [12..N] Optional Vector Geometry Payload (Present when bit 1 of flags is SET) ├── Byte 12: path_count (uint8, 2..8) ├── Bytes 13..: path_points [path_count × 4 bytes] │ ├── x (int16 LE) │ └── y (int16 LE) ├── Next Byte: branch_count (uint8, 0..3) └── Next Bytes: branch_lines [branch_count × 8 bytes] ├── x1 (int16 LE) ├── y1 (int16 LE) ├── x2 (int16 LE) └── y2 (int16 LE) [Trailing] Optional UTF-8 Street Name └── Next Bytes: street_name (up to 31 ASCII/UTF-8 bytes, null-terminated)

6.3 Maneuver Enums & Bitmask Flags

| Code | Maneuver | Description | Visual Rendering |
| --- | --- | --- | --- |
| 0 | NAV_TURN_STRAIGHT | Continue straight | Upward arrow ↑ |
| 1 | NAV_TURN_LEFT | Standard left turn | 90° left fillet curve ↰ |
| 2 | NAV_TURN_RIGHT | Standard right turn | 90° right fillet curve ⤷ |
| 3 | NAV_TURN_UTURN | U-Turn | 180° hairpin curve ⮌ |
| 4 | NAV_TURN_SLIGHT_LEFT | Fork / veer left | 45° left vector |
| 5 | NAV_TURN_SLIGHT_RIGHT | Fork / veer right | 45° right vector |
| 6 | NAV_TURN_ARRIVED | Destination reached | Chequered pin / Star icon |

7. Firmware Architecture (ESP32-S3 / LVGL 8.4)

7.1 Memory Allocation & Buffer Architecture

Display Flush Buffer: Double-buffered line buffers allocated in PSRAM:

Buffer Size: 412 × 40 pixels × 2 buffers (~66 KB PSRAM)

Render loop executed via FreeRTOS cooperative task at 5ms tick (lv_timer_handler()).

Frame Rate Target: 50–60 FPS smooth rendering with zero screen tearing via QSPI Quad-mode DMA.

7.2 Thread-Safe UI Update Pipeline

NimBLE writes execute within the BLE stack callback task.

Packet decoded into a local copy of nav_state_t.

Mutex-protected dispatch to ui_update_nav_state() updates:

Polyline point buffers for main route and branches.

Distance label formatted as "300 m" or "1.2 km".

Speed limit visibility flag and numeric label.

Progress arc angle between 35° and 145°.

Street name banner text.

8. MVP Feature Matrix & Implementation Status

| Feature Module | Specification Details | Status |
| --- | --- | --- |
| Hardware Platform | ESP32-S3R8 + Waveshare 1.46" SPD2010 QSPI LCD | Verified on Hardware (COM16) |
| I2C Bus & Expander | TCA9554 controlling LCD/TP resets at 400 kHz | Verified on Hardware |
| LVGL Rendering | 412×412 round screen layout, Montserrat typography | Verified on Hardware |
| Vector Mini-Map | 14px casing + 8px paved core + dynamic branch vectors | Verified on Hardware |
| Navigation HUD | Distance countdown, turn icons, 60px speed badge | Verified on Hardware |
| Progress Arc | 110° lower rim arc (0% to 100% completion) | Verified on Hardware |
| BLE GATT Server | NimBLE peripheral advertising "BeeLine-Moto2" | Verified on Hardware |
| Physical Button | BOOT button (GPIO0) sends BLE notification | Verified on Hardware |
| Companion App UI | MapLibre vector map + Ola Maps search & routing | Verified on Android (Galaxy A32) |
| Route Projection | Real GPS road path ➔ 412×220px vector coordinates | Verified in Companion App |
| Debug Fixture | 1-tap route test ("Kulangara Mills ➔ Foodcafe") | Verified in Companion App |

9. Performance Benchmarks & Targets

| Metric | Target | MVP Measured Result |
| --- | --- | --- |
| Display Refresh Rate | ≥ 45 FPS | 50 FPS (QSPI 40 MHz) |
| BLE Telemetry Latency | < 100 ms | < 35 ms (from GPS update to screen render) |
| BLE Stream Frequency | 5 Hz (200 ms) | 5 Hz steady |
| Packet Size | < 128 bytes | 28 – 76 bytes (typical route packet) |
| Flash Memory Usage | < 4 MB | 1.2 MB / 16 MB (7.5%) |
| SRAM Usage | < 200 KB | 112 KB / 512 KB (21.8%) |
| PSRAM Usage | < 2 MB | 184 KB / 8 MB (2.3%) |